ЛР8

В этой ЛР мы разносим разные VLAN по одному физическому интерфейсу, используя sub-interfaces (подынтерфейсы) на роутерах.

Строим схему, далее настраиваем все в соответствии с логической схемой. По сути, предыдущая лаба. Просто виртуальные подсети будут на разных роутерах, их будет несколько.

R1 ↔ R2 общаются через VLAN11

R2 ↔ R4 через VLAN12

R2 ↔ R3 через VLAN13

R3 ↔ R4 через VLAN15

R4 ↔ R5 через VLAN14

VLAN11 — R1 ↔ R2

Сеть: 10.0.11.0/30

- R1: 10.0.11.1

- R2: 10.0.11.2

VLAN12 — R2 ↔ R4

Сеть: 10.0.12.0/30

- R2: 10.0.12.1

- R4: 10.0.12.2

VLAN13 — R2 ↔ R3

Сеть: 10.0.13.0/30

- R2: 10.0.13.1

- R3: 10.0.13.2

VLAN15 — R3 ↔ R4

Сеть: 10.0.15.0/30

- R3: 10.0.15.1

- R4: 10.0.15.2

VLAN14 — R4 ↔ R5

Сеть: 10.0.14.0/30

- R4: 10.0.14.1

- R5: 10.0.14.2

Приступим

- На коммутаторе поднимем виртуальные подсети

enable

conf t

vlan 11

vlan 12

vlan 13

vlan 14

vlan 15

exit

Проверка

show interfaces trunk

- Поднимаем транк порты на коммутаторе

conf t

interface fa0/1

switchport mode trunk

switchport trunk allowed vlan 11

switchport nonegotiate

exit

interface fa0/2

switchport mode trunk

switchport trunk allowed vlan 11,12,13

switchport nonegotiate

exit

interface fa0/3

switchport mode trunk

switchport trunk allowed vlan 13,15

switchport nonegotiate

exit

interface fa0/4

switchport mode trunk

switchport trunk allowed vlan 12,14,15

switchport nonegotiate

exit

interface fa0/5

switchport mode trunk

switchport trunk allowed vlan 14

switchport nonegotiate

exit

- R1

enable

conf t

interface fa0/0.11

encapsulation dot1Q 11

ip address 10.11.0.1 255.255.255.252

exit

show ip interface brief

ping 10.11.0.2

- R2

conf t

interface fa0/0.11

encapsulation dot1Q 11

ip address 10.11.0.2 255.255.255.252

interface fa0/0.12

encapsulation dot1Q 12

ip address 10.12.0.1 255.255.255.252

interface fa0/0.13

encapsulation dot1Q 13

ip address 10.13.0.1 255.255.255.252

exit

ping 10.11.0.1

ping 10.12.0.2

ping 10.13.0.2

- R3

conf t

interface fa0/0.13

encapsulation dot1Q 13

ip address 10.13.0.2 255.255.255.252

interface fa0/0.15

encapsulation dot1Q 15

ip address 10.15.0.1 255.255.255.252

exit

ping 10.13.0.1

ping 10.15.0.2

- R4

conf t

interface fa0/0.12

encapsulation dot1Q 12

ip address 10.12.0.2 255.255.255.252

interface fa0/0.15

encapsulation dot1Q 15

ip address 10.15.0.2 255.255.255.252

interface fa0/0.14

encapsulation dot1Q 14

ip address 10.14.0.1 255.255.255.252

exit

ping 10.12.0.1

ping 10.15.0.1

ping 10.14.0.2

- R5

conf t

interface fa0/0

encapsulation dot1Q 14

ip address 10.14.0.2 255.255.255.252

exit

ping 10.14.0.1