ПЗ №10 — Полная инструкция (MAC Security + DHCP + VLAN)

Данная инструкция описывает полный процесс выполнения лабораторной работы №10:

настройка VLAN, DHCP, Port Security и проверка работы сети.

========================================

ШАГ 0 — СБОРКА ТОПОЛОГИИ

========================================

Устройства:

• Switch SW1

• Routers R1–R5

Подключения:

R1 fa0/0 → SW1 fa0/1

R2 fa0/0 → SW1 fa0/2

R3 fa0/0 → SW1 fa0/3

R4 fa0/0 → SW1 fa0/4

R5 fa1   → SW1 fa0/5

R4 fa0/1 → SW1 fa0/6

R5 fa2   → SW1 fa0/7

========================================

ШАГ 1 — VLAN И ACCESS ПОРТЫ

========================================

enable

conf t

hostname SW1

vlan 10

vlan 11

exit

interface range fa0/1 - 7

switchport mode access

spanning-tree portfast

switchport nonegotiate

exit

interface fa0/1

switchport access vlan 10

interface fa0/2

switchport access vlan 10

interface fa0/3

switchport access vlan 11

interface fa0/4

switchport access vlan 10

interface fa0/5

switchport access vlan 10

interface fa0/6

switchport access vlan 11

interface fa0/7

switchport access vlan 11

end

wr

========================================

ШАГ 2 — PORT SECURITY

========================================

fa0/1 — sticky MAC, limit 1:

interface fa0/1

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation shutdown

fa0/2 — sticky MAC, limit 1:

interface fa0/2

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation shutdown

fa0/3 — static MAC:

interface fa0/3

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address 000C.CCCC.CCCC

switchport port-security violation shutdown

end

wr

========================================

ШАГ 3 — R1/R2/R3: DHCP-КЛИЕНТЫ

========================================

interface fa0/0

ip address dhcp

no shutdown

========================================

ШАГ 4 — R5: СТАТИЧЕСКИЕ IP

========================================

interface fa1

ip address 10.0.10.5 255.255.255.0

no shutdown

interface fa2

ip address 10.0.11.5 255.255.255.0

no shutdown

========================================

ШАГ 5 — R4: DHCP-СЕРВЕР

========================================

interface fa0/0

ip address 10.0.10.4 255.255.255.0

no shutdown

interface fa0/1

ip address 10.0.11.4 255.255.255.0

no shutdown

ip dhcp excluded-address 10.0.10.1 10.0.10.10

ip dhcp pool vlan10

network 10.0.10.0 255.255.255.0

default-router 10.0.10.4

dns-server 10.0.10.4

ip dhcp pool vlan11

network 10.0.11.0 255.255.255.0

default-router 10.0.11.4

dns-server 10.0.11.4

end

wr

========================================

ШАГ 6 — ПРОВЕРКА DHCP

========================================

R1/R2/R3:

show ip interface brief

R4:

show ip dhcp binding

show ip dhcp pool

========================================

ШАГ 7 — ПРОВЕРКА СВЯЗНОСТИ

========================================

ping 10.0.10.4

ping 10.0.11.5

ping 10.0.10.5

traceroute 10.0.11.5

========================================

ШАГ 8 — ПРОВЕРКА PORT SECURITY

========================================

show port-security interface fa0/1

show port-security interface fa0/2

show port-security interface fa0/3

show interfaces status err-disabled

Восстановление порта fa0/3:

interface fa0/3

shutdown

no shutdown

========================================

Лабораторная работа завершена.

========================================