ЛР9

Отличие от 8ой в наличии маршрутизации роутеров между собой, настраиваем ее статически.

Настроим все

ШАГ 1. Настраиваем коммутатор SW1

Зайди на SW1, в CLI и вставь:

enable

conf t

hostname SW1

! --- VLAN'ы ---

vlan 11

vlan 12

vlan 13

vlan 14

vlan 15

exit

! --- Trunk к R1: VLAN 11,12 ---

interface fa0/1

switchport mode trunk

switchport trunk allowed vlan 11,12

switchport nonegotiate

no shutdown

! --- Trunk к R2: VLAN 11,13 ---

interface fa0/2

switchport mode trunk

switchport trunk allowed vlan 11,13

switchport nonegotiate

no shutdown

! --- Trunk к R3: VLAN 12,13 ---

interface fa0/3

switchport mode trunk

switchport trunk allowed vlan 12,13

switchport nonegotiate

no shutdown

! --- Access под VLAN 14 (R4 Fa0/0, R5 Fa1) ---

interface fa0/4

switchport mode access

switchport access vlan 14

no shutdown

interface fa0/5

switchport mode access

switchport access vlan 14

no shutdown

! --- Access под VLAN 15 (R4 Fa0/1, R5 Fa2) ---

interface fa0/6

switchport mode access

switchport access vlan 15

no shutdown

interface fa0/7

switchport mode access

switchport access vlan 15

no shutdown

end

wr

ШАГ 2. Настраиваем интерфейсы и loopback’и на роутерах

2.1. R1 — subinterfaces + loopback

enable

conf t

hostname R1

interface fa0/0

no shutdown

interface fa0/0.11

encapsulation dot1Q 11

ip address 10.0.0.1 255.255.255.252   ! линк к R2

interface fa0/0.12

encapsulation dot1Q 12

ip address 10.0.0.5 255.255.255.252   ! линк к R3

interface lo0

ip address 10.0.1.1 255.255.255.0     ! сеть за R1

end

wr

2.2. R2 — subinterfaces, serial к R4, loopback

enable

conf t

hostname R2

interface fa0/0

no shutdown

interface fa0/0.11

encapsulation dot1Q 11

ip address 10.0.0.2 255.255.255.252   ! к R1

interface fa0/0.13

encapsulation dot1Q 13

ip address 10.0.0.9 255.255.255.252   ! к R3

! Serial к R4 (сеть 10.0.0.12/30)

interface Serial0/1/0

description LINK_TO_R4

ip address 10.0.0.13 255.255.255.252

no shutdown

interface lo0

ip address 10.0.2.1 255.255.255.0

end

wr

2.3. R3 — subinterfaces, serial к R4, loopback’и

enable

conf t

hostname R3

interface fa0/0

no shutdown

interface fa0/0.12

encapsulation dot1Q 12

ip address 10.0.0.6 255.255.255.252   ! к R1

interface fa0/0.13

encapsulation dot1Q 13

ip address 10.0.0.10 255.255.255.252  ! к R2

! Serial к R4 (сеть 10.0.0.16/30)

interface Serial0/1/0

description LINK_TO_R4

ip address 10.0.0.17 255.255.255.252

no shutdown

! Loopback-сети за R3

interface lo0

ip address 10.0.3.1 255.255.255.128   ! 10.0.3.0/25

interface lo1

ip address 10.0.3.129 255.255.255.192 ! 10.0.3.128/26

interface lo2

ip address 10.0.3.193 255.255.255.224 ! 10.0.3.192/27

interface lo3

ip address 10.0.3.225 255.255.255.224 ! 10.0.3.224/27

end

wr

2.4. R4 — FastEthernet к SW1, два serial’а, loopback

enable

conf t

hostname R4

! К SW1, VLAN 14

interface fa0/0

ip address 10.0.0.21 255.255.255.252   ! сеть 10.0.0.20/30

no shutdown

! К SW1, VLAN 15

interface fa0/1

ip address 10.0.0.25 255.255.255.252   ! сеть 10.0.0.24/30

no shutdown

! Serial к R3 (10.0.0.16/30)

interface Serial0/1/0

description LINK_TO_R3

ip address 10.0.0.18 255.255.255.252

clock rate 64000      ! если этот порт DCE

no shutdown

! Serial к R2 (10.0.0.12/30)

interface Serial0/1/1

description LINK_TO_R2

ip address 10.0.0.14 255.255.255.252

clock rate 64000      ! если этот порт DCE

no shutdown

interface lo0

ip address 10.0.4.1 255.255.255.0

end

wr

2.5. R5 — два линка к SW1, loopback’и

enable

conf t

hostname R5

! К SW1, VLAN 14

interface fa1

ip address 10.0.0.22 255.255.255.252   ! пара к 10.0.0.21 (R4)

no shutdown

! К SW1, VLAN 15

interface fa2

ip address 10.0.0.26 255.255.255.252   ! пара к 10.0.0.25 (R4)

no shutdown

! Loopback-сети за R5

interface lo0

ip address 10.0.5.1 255.255.255.128    ! 10.0.5.0/25

interface lo1

ip address 10.0.5.129 255.255.255.192  ! 10.0.5.128/26

interface lo2

ip address 10.0.5.193 255.255.255.192  ! 10.0.5.192/26

end

wr

ШАГ 3. Статическая маршрутизация

Теперь учим роутеры ходить друг к другу.

3.1. R5 — два default’а (основной + резерв)

enable

conf t

! основной маршрут по VLAN 15 через R4

ip route 0.0.0.0 0.0.0.0 10.0.0.25

! резерв по VLAN 14 через R4, AD=5

ip route 0.0.0.0 0.0.0.0 10.0.0.21 5

end

wr

3.2. R4 — сети R5 + все остальные

enable

conf t

! --- Сети за R5 (основной путь через VLAN15, резерв через VLAN14) ---

ip route 10.0.5.0   255.255.255.128 10.0.0.26

ip route 10.0.5.0   255.255.255.128 10.0.0.22 5

ip route 10.0.5.128 255.255.255.192 10.0.0.26

ip route 10.0.5.128 255.255.255.192 10.0.0.22 5

ip route 10.0.5.192 255.255.255.192 10.0.0.26

ip route 10.0.5.192 255.255.255.192 10.0.0.22 5

! --- сети R1 и R2 через R2 (10.0.0.13) ---

ip route 10.0.1.0 255.255.255.0 10.0.0.13

ip route 10.0.2.0 255.255.255.0 10.0.0.13

! --- сети R3 через R3 (10.0.0.17) ---

ip route 10.0.3.0   255.255.255.128 10.0.0.17

ip route 10.0.3.128 255.255.255.192 10.0.0.17

ip route 10.0.3.192 255.255.255.224 10.0.0.17

ip route 10.0.3.224 255.255.255.224 10.0.0.17

end

wr

3.3. R1 — всё, что не он

enable

conf t

! сети R2

ip route 10.0.2.0 255.255.255.0 10.0.0.2

! сети R3

ip route 10.0.3.0   255.255.255.128 10.0.0.6

ip route 10.0.3.128 255.255.255.192 10.0.0.6

ip route 10.0.3.192 255.255.255.224 10.0.0.6

ip route 10.0.3.224 255.255.255.224 10.0.0.6

! сети R4 и R5 через R2

ip route 10.0.4.0   255.255.255.0   10.0.0.2

ip route 10.0.5.0   255.255.255.128 10.0.0.2

ip route 10.0.5.128 255.255.255.192 10.0.0.2

ip route 10.0.5.192 255.255.255.192 10.0.0.2

end

wr

3.4. R2 — всё через R1, R3, R4

enable

conf t

! R1

ip route 10.0.1.0 255.255.255.0 10.0.0.1

! R3

ip route 10.0.3.0   255.255.255.128 10.0.0.10

ip route 10.0.3.128 255.255.255.192 10.0.0.10

ip route 10.0.3.192 255.255.255.224 10.0.0.10

ip route 10.0.3.224 255.255.255.224 10.0.0.10

! R4 и R5 через serial к R4

ip route 10.0.4.0   255.255.255.0   10.0.0.14

ip route 10.0.5.0   255.255.255.128 10.0.0.14

ip route 10.0.5.128 255.255.255.192 10.0.0.14

ip route 10.0.5.192 255.255.255.192 10.0.0.14

end

wr

3.5. R3 — всё через R1, R2, R4

enable

conf t

! R1

ip route 10.0.1.0 255.255.255.0 10.0.0.6

! R2

ip route 10.0.2.0 255.255.255.0 10.0.0.10

! R4 и R5 через serial к R4

ip route 10.0.4.0   255.255.255.0   10.0.0.18

ip route 10.0.5.0   255.255.255.128 10.0.0.18

ip route 10.0.5.128 255.255.255.192 10.0.0.18

ip route 10.0.5.192 255.255.255.192 10.0.0.18

end

wr

ШАГ 4. Проверка (минимум, который надо сделать)

4.1. На каждом маршрутизаторе

show ip interface brief

show ip route

Все нужные интерфейсы должны быть up/up.
В таблице маршрутизации увидишь кучу S и S*.

4.2. Пинги

С R1:

ping 10.0.2.1

ping 10.0.3.1

ping 10.0.4.1

ping 10.0.5.1

ping 10.0.5.129

ping 10.0.5.193

С R5:

ping 10.0.1.1

ping 10.0.2.1

ping 10.0.3.1

ping 10.0.4.1

Должно быть везде !!!!!.